LIEBHERR 71 EC-B 5

LOAD CHART PRACTICE - INSTRUCTOR ANSWER KEY

Use with the revised 6-question practice sheet

1. Gross Load / Capacity

| Given | 2-part • 50.0 m jib • 35.0 m radius • 1,400 kg load • 125 kg rigging |
| --- | --- |
| Gross suspended load | 1,400 + 125 = 1,525 kg |
| Chart capacity | 50.0 m jib @ 35.0 m radius = 1,580 kg |
| Result | YES — lift is within rated capacity. |
| Remaining capacity | 1,580 - 1,525 = 55 kg |

2. Maximum Radius

| Given | 2-part • 45.0 m jib • 1,700 kg load • 90 kg rigging |
| --- | --- |
| Gross suspended load | 1,700 + 90 = 1,790 kg |
| Check 35.0 m | Rated capacity = 1,770 kg — NOT enough |
| Check 32.5 m | Rated capacity = 1,940 kg — permitted |
| Answer | Maximum listed hook radius = 32.5 m |

3. Different Jib Configurations

| Given | 2-part • 35.0 m radius |
| --- | --- |
| 50.0 m jib | Rated capacity = 1,580 kg |
| 40.0 m jib | Rated capacity = 1,880 kg |
| Calculation | 1,880 - 1,580 = 300 kg |
| Answer | The 40.0 m jib provides 300 kg more rated capacity. |

4. Available Capacity

| Given | 4-part • 47.5 m jib • 40.0 m radius • 1,100 kg load • 100 kg rigging |
| --- | --- |
| Gross suspended load | 1,100 + 100 = 1,200 kg |
| Chart capacity | 47.5 m jib @ 40.0 m radius = 1,290 kg |
| Calculation | 1,290 - 1,200 = 90 kg |
| Answer | 90 kg additional gross load capacity remains. |

5. Maximum Allowable Load

| Given | 4-part • 42.5 m jib • 37.5 m radius • 150 kg rigging |
| --- | --- |
| Chart capacity | 42.5 m jib @ 37.5 m radius = 1,550 kg |
| Calculation | 1,550 - 150 = 1,400 kg |
| Answer | Maximum allowable load weight, excluding rigging = 1,400 kg |

6. Between Listed Radii

| Given | 2-part • 50.0 m jib • actual radius 36.0 m • 1,300 kg load • 100 kg rigging |
| --- | --- |
| Radius rule | 36.0 m is between listed radii. Use the next larger listed radius: 37.5 m. |
| Gross suspended load | 1,300 + 100 = 1,400 kg |
| Chart capacity | 50.0 m jib @ 37.5 m radius = 1,450 kg |
| Result | YES — permitted. Remaining capacity = 1,450 - 1,400 = 50 kg |

Instructor note: working hook-radius columns are used for capacity lookup; the separate jib-end “r” dimension is not the load radius.