POTAIN MDT 218 A J10

LOAD CHART WRITTEN TEST - INSTRUCTOR ANSWER KEY

1. Maximum Working Radius

| Gross suspended load | 3.15 + 0.20 = 3.35 t |
| --- | --- |
| 40 m capacity | 3.40 t - permitted |
| 42 m capacity | 3.20 t - not sufficient |
| Answer | B - 40 m |

2. Jib Configuration Comparison

| 65 m jib @ 40 m | 3.40 t |
| --- | --- |
| 50 m jib @ 40 m | 4.50 t |
| Difference | 4.50 - 3.40 = 1.10 t |
| Answer | B - 1.10 t additional gross capacity |

3. Maximum Allowable Load

| Chart capacity | 60 m jib @ 45 m radius = 3.40 t |
| --- | --- |
| Rigging | 0.25 t |
| Maximum load | 3.40 - 0.25 = 3.15 t |
| Answer | A - 3.15 t |

4. Between Listed Radii

| Actual radius | 43.0 m |
| --- | --- |
| Radius rule | Use next larger listed radius: 45 m |
| Gross suspended load | 3.35 + 0.20 = 3.55 t |
| Chart capacity | 55 m jib @ 45 m = 3.90 t |
| Remaining | 3.90 - 3.55 = 0.35 t |
| Answer | YES - with 0.35 t remaining |
| Test issue | The current multiple-choice answers do not include the correct 0.35 t remaining. |

5. Correct Chart Selection / Configuration

| Required chart | Lower supplied Potain load chart |
| --- | --- |
| Chart capacity | 50 m jib @ 40 m radius = 4.50 t |
| Answer | B - 4.50 t |

6. Configuration Difference

| Upper chart | 45 m jib @ 40 m = 4.90 t |
| --- | --- |
| Lower chart | 45 m jib @ 40 m = 4.70 t |
| Difference | 4.90 - 4.70 = 0.20 t |
| Answer | B - 0.20 t |

7. Structural Capacity Check

| Gross suspended load | 2.60 + 0.25 = 2.85 t |
| --- | --- |
| Lower-chart capacity | 60 m jib @ 50 m = 2.95 t |
| Remaining | 2.95 - 2.85 = 0.10 t |
| Answer | A - Yes, with 0.10 t remaining |

8. Hoist Performance

| Gross suspended load | 2.25 + 0.20 = 2.45 t |
| --- | --- |
| Hoist ratings | 2.5 t @ 70 m/min can carry 2.45 t |
| Answer | C - 70 m/min |

9. Reeving / Hoist Configuration

| Gross suspended load | 4.80 t |
| --- | --- |
| 50 m/min rating | 2.5 t - insufficient |
| 35 m/min rating | 5.0 t - sufficient |
| Answer | C - 35 m/min |

10. Power and Mechanism Information

| Electrical supply | 400 V / 50 Hz |
| --- | --- |
| Apparent power | 58 kVA |
| Answer | B - 400 V, 50 Hz, 58 kVA |

11. Relevant vs. Irrelevant Information

| Relevant for lookup | 40 m jib • 35 m radius • upper chart • 0.20 t rigging |
| --- | --- |
| Irrelevant to capacity lookup | 400 V / 50 Hz • 37 kW • 7 DVF 4 |
| Chart capacity | 40 m jib @ 35 m radius = 5.40 t |
| Maximum load | 5.40 - 0.20 = 5.20 t |
| Answer | D - 5.20 t |

12. Final Multi-Chart Problem

| a) Gross suspended load | 2.45 + 0.20 = 2.65 t |
| --- | --- |
| b) Structural capacity | 65 m jib @ 45 m radius = 2.80 t |
| c) Lift permitted? | YES |
| d) Remaining capacity | 2.80 - 2.65 = 0.15 t |
| e) Highest hoisting speed | 2.65 t is above the 2.5 t @ 70 rating; use 3.75 t @ 49 m/min |
| Answer | YES • 0.15 t remaining • 49 m/min |

Instructor correction note: Question 4 needs its answer choices revised before the written test is issued. The charted result is YES, with 0.35 t remaining.