LIEBHERR 71 EC-B 5

LOAD CHART PRACTICE - QUESTION SHEET

| Name: __________________________ | Date: ______________ | Score: __________ |
| --- | --- | --- |

Use the supplied Liebherr 71 EC-B 5 radius-and-capacity chart. Select the stated parts-of-line chart for each question. Use the working hook-radius columns across the chart; do not use the separate jib-end r dimension as the load radius. Show calculations where required.

1. Gross Load / Capacity

Parts of line: 2-part

Jib length: 50.0 m

Hook radius: 35.0 m

Load weight: 1,400 kg

Rigging weight: 125 kg

Determine whether the lift is within the crane's rated capacity.

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

2. Maximum Radius

Parts of line: 2-part

Jib length: 45.0 m

Load weight: 1,700 kg

Rigging weight: 90 kg

Determine the maximum listed hook radius at which the load may be lifted.

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

3. Different Jib Configurations

Parts of line: 2-part

Hook radius: 35.0 m

Jib configuration A: 50.0 m

Jib configuration B: 40.0 m

Determine the difference in rated capacity between the two jib configurations.

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

4. Available Capacity

Parts of line: 4-part

Jib length: 47.5 m

Hook radius: 40.0 m

Load weight: 1,100 kg

Rigging weight: 100 kg

Determine how much additional gross load capacity remains.

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

5. Maximum Allowable Load

Parts of line: 4-part

Jib length: 42.5 m

Hook radius: 37.5 m

Rigging weight: 150 kg

Determine the maximum allowable load weight, excluding the rigging.

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

6. Between Listed Radii

Parts of line: 2-part

Jib length: 50.0 m

Actual hook radius: 36.0 m

Load weight: 1,300 kg

Rigging weight: 100 kg

Determine whether the lift is permitted using the supplied load chart.

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________