TOWER CRANE CLIMBING
& RECONFIGURATION

Red Seal Task C-7 Practice Test — Instructor Answer Key

30 Questions | Suggested Pass: 70% (21/30)

Use: Instructor teaching key. Explanations are written to support the answer slides. These are original Red Seal-style practice questions, not official Red Seal examination questions.

Quick Answer Key

| 1. B | 2. C | 3. A | 4. D | 5. C | 6. C | 7. B | 8. A | 9. C | 10. B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11. A | 12. A | 13. C | 14. A | 15. C | 16. B | 17. C | 18. B | 19. A | 20. C |
| 21. B | 22. C | 23. A | 24. C | 25. B | 26. C | 27. B | 28. C | 29. C | 30. C |

Part A — BC Regulations & Climbing Requirements

QUESTION 1 — CORRECT ANSWER: B Why: Erection, climbing, repositioning and dismantling must be done by qualified persons and in accordance with the crane manufacturer's instructions, or a professional engineer's instructions if the installation varies from the manufacturer's instructions. Why the other choices are wrong: A certified operator is not automatically qualified to perform a particular climb; an OEM representative is not universally required; supervision by an operator does not replace the qualified-person requirement. Reference: WorkSafeBC OHSR 14.73.2(1)

QUESTION 2 — CORRECT ANSWER: C Why: The activity must be directed by a qualified supervisor who is qualified for that particular tower crane and activity. Why the other choices are wrong: The prime contractor, operator or site superintendent may have other responsibilities, but the regulation specifically requires a qualified supervisor. Reference: WorkSafeBC OHSR 14.73.1 and 14.73.2(2)

QUESTION 3 — CORRECT ANSWER: A Why: If the installation varies from the crane manufacturer's instructions, the work must be carried out in accordance with instructions from a professional engineer. Why the other choices are wrong: GC or operator approval does not replace engineering instructions, and adding another supervisor does not authorize a deviation. Reference: WorkSafeBC OHSR 14.73.2(1)(b)

QUESTION 4 — CORRECT ANSWER: D Why: Climbing is a defined crane activity requiring a Notice of Project. The applicable WorkSafeBC NOP requirement is at least two weeks before the crane activity begins. Why the other choices are wrong: The shorter notice periods in A-C do not satisfy the normal tower-crane NOP requirement. Reference: WorkSafeBC OHSR 14.73.3; WorkSafeBC NOP-TC requirements

QUESTION 5 — CORRECT ANSWER: C Why: The applicable climbing instructions must control the operation. BC Crane Safety also expects operators to locate and interpret manufacturer raising procedures, sequence, balance requirements, component inspections and wind limitations. Why the other choices are wrong: Experience or a similar crane's manual is not a substitute for the applicable instructions. Reference: WorkSafeBC OHSR 14.73.2 and 14.79; BC Crane Safety Tower Crane Operator Standard N2

QUESTION 6 — CORRECT ANSWER: C Why: Before use after climbing, the qualified supervisor must verify compliance with the applicable specifications; required P.Eng. certification applies to deviations; a qualified person adjusts the overload-prevention system as necessary; required records/certifications must be available. Why the other choices are wrong: A normal pre-use check, reduced-capacity operation, or an unloaded function test alone does not satisfy 14.75. Reference: WorkSafeBC OHSR 14.75

Part B — Bottom Climbing | C-7.01

QUESTION 7 — CORRECT ANSWER: B Why: Bottom climbing raises the crane/mast using the building-related support system, including beams/collars as applicable, rather than inserting new tower sections immediately below the upper works. Why the other choices are wrong: A describes top climbing; C and D are not climbing methods. Reference: Red Seal RSOS 2023, Task C-7 / C-7.01

QUESTION 8 — CORRECT ANSWER: A Why: Bottom climbing depends on engineered building support locations and structural capacity. These support arrangements must match the applicable engineering requirements. Why the other choices are wrong: Paint condition, seat adjustment and trolley speed are not the primary structural prerequisite being tested. Reference: Red Seal RSOS C-7.01; WorkSafeBC OHSR 14.74

QUESTION 9 — CORRECT ANSWER: C Why: A support arrangement that does not match the engineering design must be resolved before the climb proceeds. Shoring, bracing and building connections must be constructed as designed and certified as required. Why the other choices are wrong: Unloading the hook, improvising material or slowing the climb does not correct an engineering discrepancy. Reference: WorkSafeBC OHSR 14.74; Red Seal RSOS C-7.01

QUESTION 10 — CORRECT ANSWER: B Why: Unexpected movement at a structural support during load transfer is an abnormal condition. Stop, establish the required safe condition and investigate before continuing. Why the other choices are wrong: More hydraulic force, continuing to another level, or improvising support can worsen an unstable or improperly supported condition. Reference: Red Seal RSOS C-7.01; manufacturer/engineering procedure

Part C — Top Climbing | C-7.02

QUESTION 11 — CORRECT ANSWER: A Why: The RSOS identifies climbing-system components such as the climbing unit, secured dogs, electrical/hydraulic components, collars and beams. BC Crane Safety similarly identifies climbing frame, hydraulic/electrical components, tie-ins and safety devices. Why the other choices are wrong: The other choices contain mobile/crawler crane components rather than top-climbing tower-crane components. Reference: Red Seal RSOS C-7.02; BC Crane Safety Tower Crane Operator Standard N2

QUESTION 12 — CORRECT ANSWER: A Why: The RSOS explicitly requires knowledge of top-climbing procedures and the importance of the climbing sequence. Why the other choices are wrong: The other topics are outside the C-7.02 top-climbing knowledge requirement. Reference: Red Seal RSOS C-7.02.02L

QUESTION 13 — CORRECT ANSWER: C Why: Required shoring must be in place and meet the applicable engineered/manufacturer requirements before climbing begins. Why the other choices are wrong: An unloaded hook or reduced hydraulic pressure does not compensate for missing structural support. Reference: Red Seal RSOS C-7.02; WorkSafeBC OHSR 14.74

QUESTION 14 — CORRECT ANSWER: A Why: The RSOS specifically identifies wind, freezing rain, temperature, snow, lightning and fog as weather conditions affecting climbing. Why the other choices are wrong: The other choices omit listed hazards or substitute conditions not given in this RSOS list. Reference: Red Seal RSOS C-7.02.02L

QUESTION 15 — CORRECT ANSWER: C Why: The climbing limit must come from the applicable manufacturer climbing instructions/specifications. WorkSafeBC's 50 km/h provision is a fallback for crane operation when the manufacturer has not specified an operating limit; it is not a universal climbing limit. Why the other choices are wrong: There is no universal BC 37 km/h climbing rule, yesterday's wind is irrelevant to the permitted limit, and the 50 km/h fallback should not be substituted for a manufacturer climbing limit. Reference: Red Seal RSOS C-7.02; WorkSafeBC OHSR 14.73.2 and 14.92; BC Crane Safety N2

QUESTION 16 — CORRECT ANSWER: B Why: Tower-crane operations must stop below -18 C (0 F) unless otherwise specified by the manufacturer or a professional engineer. Why the other choices are wrong: The regulation does not create a half-capacity rule, does not limit the requirement to climbing only, and does not make hydraulic-oil temperature the deciding criterion. Reference: WorkSafeBC OHSR 14.93

QUESTION 17 — CORRECT ANSWER: C Why: Lightning is an identified climbing weather hazard. When thunder is heard, personnel should be removed from exposure and the operation stopped/secured as required. Canadian lightning guidance says to seek safe shelter when thunder is heard. Why the other choices are wrong: Waiting for visible lightning, finishing the section regardless of conditions, or relying on crane grounding does not adequately protect exposed workers. Reference: Red Seal RSOS C-7.02 weather knowledge; Environment and Climate Change Canada lightning safety guidance

QUESTION 18 — CORRECT ANSWER: B Why: Correct balance controls the forces acting through the climbing system and is essential to maintaining the intended stable climbing condition. Why the other choices are wrong: Balance does not increase rated capacity or hydraulic speed, and preventing normal trolley movement is not its purpose. Reference: Red Seal RSOS C-7.02; BC Crane Safety N2

QUESTION 19 — CORRECT ANSWER: A Why: The operator must establish the balance configuration specified for that crane and climbing procedure, whether by trolley position, luffing geometry or another manufacturer-defined method. Why the other choices are wrong: Generic minimum radius, another crane's setup, or hydraulic pressure is not a substitute for the specified balance procedure. Reference: Red Seal RSOS C-7.02; BC Crane Safety N2; applicable manufacturer procedure

QUESTION 20 — CORRECT ANSWER: C Why: If the required balance cannot be established, the climb must not continue until the cause is resolved according to the applicable procedure. Why the other choices are wrong: Increasing pressure, adding unapproved counterweight or continuing while out of balance can introduce unsafe forces and instability. Reference: Red Seal RSOS C-7.02; applicable manufacturer procedure

Part D — Tower Crane Reconfiguration | C-7.03

QUESTION 21 — CORRECT ANSWER: B Why: The RSOS specifically requires verification that NDT inspection of additional components has been completed and documented before operation. Why the other choices are wrong: The RSOS does not state that every tower bolt must receive magnetic inspection, nor does an annual inspection or prime-contractor daily inspection replace the specified NDT verification. Reference: Red Seal RSOS C-7.03.02P

QUESTION 22 — CORRECT ANSWER: C Why: Following reconfiguration, the LMI must be reconfigured and the applicable limits verified and tested for the new configuration. Why the other choices are wrong: Leaving old settings, applying an arbitrary derating, or resetting only one limit does not establish correct protection for the new configuration. Reference: Red Seal RSOS C-7.03.11P

QUESTION 23 — CORRECT ANSWER: A Why: The operator must verify that the configuration is acceptable to the crane manufacturer or professional engineer and verify that required reconfiguration permits have been obtained. Why the other choices are wrong: Seniority, schedule, a previous load chart, or hook height alone do not meet these performance criteria. Reference: Red Seal RSOS C-7.03.01P and C-7.03.03P

QUESTION 24 — CORRECT ANSWER: C Why: An LMI showing the previous configuration is not correctly configured for the crane as built. Do not operate until the LMI and applicable limits are correctly set, verified and tested. Why the other choices are wrong: Arbitrary derating, ignoring the LMI, or operating unloaded does not correct the configuration mismatch. Reference: Red Seal RSOS C-7.03.11P

Part E — Integrated Red Seal-Style Scenarios

QUESTION 25 — CORRECT ANSWER: B Why: The RSOS requires weather to be checked before starting and continually monitored as it changes. Being below a numerical limit at one instant does not justify starting when conditions are rapidly deteriorating. Why the other choices are wrong: Rushing the climb, increasing speed or considering only average wind ignores changing/gust conditions. Reference: Red Seal RSOS C-7.02 weather monitoring; manufacturer limits

QUESTION 26 — CORRECT ANSWER: C Why: Misalignment must be investigated and corrected using the applicable procedure. Structural components should not be forced into an unknown or incorrect condition. Why the other choices are wrong: Forcing, partially connecting, or uncontrolled striking can damage components or create an improper structural connection. Reference: Red Seal RSOS C-7.02; applicable manufacturer procedure

QUESTION 27 — CORRECT ANSWER: B Why: Required connections must be completed and verified before moving to a stage that depends on them for the intended load path/support condition. Why the other choices are wrong: Continuing, relying on the hydraulic system, or an inadequate remote visual check does not establish the required connection. Reference: Red Seal RSOS C-7.02; applicable manufacturer sequence

QUESTION 28 — CORRECT ANSWER: C Why: Experience does not authorize an undocumented change to the climbing procedure. Follow the manufacturer; where the installation varies, professional-engineer instructions are required. Why the other choices are wrong: Crew agreement or reduced pressure cannot authorize a deviation. Reference: WorkSafeBC OHSR 14.73.2(1)

QUESTION 29 — CORRECT ANSWER: C Why: The RSOS requires radio/cell communication with the erection crew throughout climbing. If required communication is lost, the operation must be stopped safely until communication is restored. Why the other choices are wrong: Incomplete visual hand signals, finishing a stroke without communication, or the supervisor's presence does not replace the required communication system. Reference: Red Seal RSOS C-7.01/C-7.02 communication performance criteria

QUESTION 30 — CORRECT ANSWER: C Why: Physical completion of the climb is not authorization to return to service. The required verification, applicable engineering certifications, overload-prevention-system adjustments and documentation must be completed first. Why the other choices are wrong: Reduced capacity, a single test lift or operator discretion does not replace the regulatory before-use requirements. Reference: WorkSafeBC OHSR 14.75

Primary source set

• Red Seal Occupational Standard — Tower Crane Operator (2023), Task C-7.

• WorkSafeBC OHS Regulation Part 14 — sections 14.73.1–14.75, 14.79, 14.92 and 14.93 as applicable.

• BC Crane Safety — Tower Crane Operator Standard, climbing-crane competency N1/N2 and weather knowledge.

• Environment and Climate Change Canada — lightning safety guidance (supplemental for Q17).