TOWER CRANE CLIMBING
& RECONFIGURATION

Red Seal Task C-7 Practice Test

Student Written Test

| Name: ____________________________________ | Date: _____________________________________ |
| --- | --- |
| Instructor: _________________________________ | Score: __________ / 30 ________ % |

Instructions

Select the BEST answer for each question. Mark one answer only. These are original Red Seal-style practice questions and are not official Red Seal examination questions.

Suggested passing score: 70% (21/30).

Part A - BC Regulations & Climbing Requirements

1. In British Columbia, erection, climbing, repositioning and dismantling of a tower crane must be performed: A. By any certified tower crane operator B. By qualified persons in accordance with the manufacturer's instructions, or professional engineer's instructions where applicable C. Only by a manufacturer's representative D. By any worker supervised by the crane operator Answer: A B C D

2. Who must direct a tower crane climbing operation? A. The prime contractor B. The crane operator C. A qualified supervisor D. The site superintendent Answer: A B C D

3. A proposed tower crane installation varies from the manufacturer's instructions. What is required before the climbing operation proceeds? A. Written instructions from a professional engineer B. Permission from the general contractor C. Approval from the crane operator D. A second qualified supervisor Answer: A B C D

4. A Notice of Project for tower crane climbing must normally be submitted to WorkSafeBC: A. 24 hours before the climb B. 72 hours before the climb C. One week before the climb D. At least two weeks before the climb Answer: A B C D

5. The climbing crew arrives at the crane, but the applicable manufacturer's climbing instructions are not available at the workplace. What should happen? A. Use the instructions for a similar crane B. Continue if the supervisor has climbed this model previously C. Obtain the applicable instructions before climbing proceeds D. Allow the most experienced crew member to determine the sequence Answer: A B C D

6. A tower crane has been climbed. Which BEST describes what is required before it is returned to service? A. Only the operator's normal pre-use inspection is required B. The crane may operate below 50% capacity until documentation is completed C. Required supervisor verification, applicable engineering certification, overload-prevention-system adjustment and required documentation must be completed D. An unloaded hoist, trolley and slew test is sufficient Answer: A B C D

Part B - Bottom Climbing (RSOS C-7.01)

7. Which statement BEST describes bottom climbing? A. New tower sections are inserted immediately below the upper works B. The crane and mast are raised and supported using beams/collars associated with the surrounding building structure C. The jib is raised to increase hook height D. The crane climbs by increasing counterweight Answer: A B C D

8. Before a bottom-climbing operation, which condition is particularly important to verify? A. Building support locations and structural requirements B. Hook-block paint condition C. Operator seat adjustment D. Trolley travel speed Answer: A B C D

9. Engineering documents require a specific climbing-beam/support arrangement. The actual installation does not match the engineering drawings. What should happen? A. Continue if the crane is unloaded B. Modify the arrangement using available material C. Stop and have the discrepancy resolved in accordance with the engineering requirements D. Reduce climbing speed Answer: A B C D

10. During a bottom climb, unexpected movement occurs at a structural support point while load is being transferred. What is the BEST action? A. Increase hydraulic pressure to complete the transfer B. Stop the operation, establish a safe condition and investigate the movement C. Continue until the next support level is reached D. Add temporary material beneath the support while continuing the climb Answer: A B C D

Part C - Top Climbing (RSOS C-7.02)

11. Which group contains components associated with top-climbing systems? A. Climbing unit, secured dogs, hydraulic components, collars and beams B. Outrigger floats, axle locks and boom dolly C. Travel brakes, steering cylinders and tires D. Crawler tracks, boom stops and gantry Answer: A B C D

12. Which of the following is specifically identified in the Red Seal Occupational Standard as required top-climbing knowledge? A. Importance of the climbing sequence B. Concrete pump output C. Mobile-crane outrigger configuration D. Truck axle loading Answer: A B C D

13. Before top climbing begins, required shoring has not been completed. What should happen? A. Begin the climb and complete the shoring later B. Proceed because the hook is unloaded C. Do not begin until the required shoring is in place and meets the applicable requirements D. Reduce hydraulic pressure during the climb Answer: A B C D

14. Which group contains weather conditions specifically identified in the RSOS as affecting climbing? A. Wind, freezing rain, temperature, snow, lightning and fog B. Wind and temperature only C. Rain, humidity and barometric pressure D. Wind, UV index and humidity Answer: A B C D

15. Before top climbing, where should the crew determine the applicable maximum climbing wind speed? A. A universal WorkSafeBC climbing limit of 37 km/h B. The previous day's recorded wind speed C. The applicable manufacturer's climbing instructions/specifications D. WorkSafeBC's general 50 km/h fallback operating limit Answer: A B C D

16. The ambient temperature drops below -18 C (0 F) during tower crane operations in British Columbia. What applies? A. Operations may continue at half capacity B. Operations must stop unless otherwise specified by the manufacturer or a professional engineer C. Only climbing must stop D. Operations may continue if hydraulic-oil temperature remains above 0 C Answer: A B C D

17. Thunder is heard while a climbing operation is underway. What is the BEST response? A. Continue until lightning becomes visible B. Complete the mast section before stopping C. Stop and secure the operation as required and have personnel seek appropriate shelter D. Continue because the tower crane is grounded Answer: A B C D

18. Why is crane balance critical during top climbing? A. It increases the crane's rated hook capacity B. It controls forces acting on the climbing system and helps maintain stability C. It increases hydraulic-system speed D. It prevents normal trolley movement Answer: A B C D

19. The manufacturer specifies a particular trolley position or luffing configuration to establish climbing balance. What should the operator do? A. Establish the manufacturer's specified configuration B. Position the hook at minimum radius regardless of the procedure C. Use the configuration from the previous crane climbed D. Select whichever position results in the lowest hydraulic pressure Answer: A B C D

20. While establishing the climbing condition, the crane cannot be balanced as required by the manufacturer's procedure. What should happen? A. Increase hydraulic pressure B. Add temporary counterweight C. Stop and resolve the balance condition in accordance with the applicable procedure D. Continue slowly until the next support point is reached Answer: A B C D

Part D - Tower Crane Reconfiguration (RSOS C-7.03)

21. Before additional components are placed into service during a tower crane reconfiguration, what inspection does the RSOS specifically require the operator to verify has been completed? A. Magnetic inspection of every tower bolt B. Required non-destructive testing (NDT) of the additional components C. Annual inspection of the entire crane D. Daily inspection by the prime contractor Answer: A B C D

22. A tower crane has been reconfigured. What must be done with the LMI and applicable limits before operation in the new configuration? A. Leave the previous settings until the next annual inspection B. Reduce all displayed capacities by 20% C. Reconfigure the LMI and verify and test the applicable limits for the new configuration D. Reset only the maximum-radius limit Answer: A B C D

23. Before tower crane reconfiguration begins, which combination must be verified? A. The configuration is acceptable to the manufacturer or professional engineer, and required permits have been obtained B. Operator seniority and the project schedule C. The previous load chart and last daily inspection only D. Hook height and concrete strength only Answer: A B C D

24. After reconfiguration, the LMI still displays the crane's previous jib configuration. What should the operator do? A. Operate at 50% of displayed capacity B. Use the paper load chart and ignore the LMI C. Do not operate until the LMI is correctly reconfigured and applicable limits are verified and tested D. Continue if no load is on the hook Answer: A B C D

Part E - Integrated Red Seal-Style Scenarios

25. A top climb is scheduled. Wind remains below the manufacturer's maximum climbing limit, but gusts are increasing rapidly. What is the BEST action? A. Begin immediately before the wind becomes stronger B. Evaluate the changing conditions and do not proceed unless the operation can remain within applicable limits and safe procedures C. Increase climbing speed D. Use only the average wind reading Answer: A B C D

26. During a top climb, a mast section does not align correctly with its required connection points. What should the crew do? A. Use the hydraulic climbing system to force the section into alignment B. Install the connections that align and correct the others later C. Stop, determine the cause and follow the applicable manufacturer's procedure D. Strike the section until it aligns Answer: A B C D

27. A climbing stage has been completed, but one required structural connection cannot be verified. The crew is ready for the next stage. What should happen? A. Continue and verify the connection afterward B. Do not continue until the required connection is correctly completed and verified C. Continue while the hydraulic system carries the load D. Have the operator visually check the connection from the cab Answer: A B C D

28. A qualified climbing supervisor proposes changing the manufacturer's climbing sequence to save time. The change is not covered by the manufacturer's instructions. What is the BEST response? A. Follow the supervisor because experience overrides the manual B. Proceed if the entire climbing crew agrees C. Do not deviate from the applicable instructions unless the variation is properly addressed by professional-engineer instructions as required D. Test the modified sequence using reduced hydraulic pressure Answer: A B C D

29. During a climbing operation, required radio communication between the operator and climbing crew is lost. What should happen? A. Continue using hand signals even though the entire crew cannot maintain visual contact B. Complete the current climbing stroke before stopping C. Stop the operation safely until the required communication is restored D. Continue if the qualified supervisor remains on the crane Answer: A B C D

30. The physical climbing operation is complete and the crane appears normal. Lifting operations are waiting. What is the BEST action? A. Resume lifting below 50% capacity B. Perform one test lift and return to normal operation C. Complete the required post-climb verification, applicable certifications, adjustments and documentation before returning the crane to service D. Allow the operator to determine whether additional verification is necessary Answer: A B C D

END OF TEST