TEREX CTT 91-5

LOAD CHART & MECHANISMS TEST - INSTRUCTOR ANSWER KEY

1. Highest Permitted Hoisting Speed

| Gross suspended load | 2.35 t |
| --- | --- |
| Hoist configuration | 2.5 t configuration |
| Applicable rating | 2.5 t @ 34 m/min |
| Answer | A - 34 m/min |

2. Power Supply / Mechanism Identification

| Manufacturer data | 18 AFC 25 mechanism: 400 V / 50 Hz or 460 V / 60 Hz |
| --- | --- |
| Answer | C - 400 V / 50 Hz or 460 V / 60 Hz |

3. Structural Capacity

| Gross suspended load | 1.85 + 0.15 = 2.00 t |
| --- | --- |
| Chart capacity | 50 m jib • 35 m radius • 5 t row = 2.04 t |
| Remaining | 2.04 - 2.00 = 0.04 t = 40 kg |
| Answer | A - Yes, with 40 kg remaining |

4. Maximum Working Radius

| Gross suspended load | 2.40 t |
| --- | --- |
| 30 m capacity | 2.45 t - permitted |
| 35 m capacity | 2.04 t - not sufficient |
| Answer | C - 30 m |

5. Correct Capacity Row

| Configuration | 50 m jib • 40 m radius • 2.5 t / 30.63 m row |
| --- | --- |
| Chart capacity | 1.73 t |
| Answer | B - 1.73 t |

6. Same Crane - Different Configuration

| 2.5 t / 30.63 m row | 1.73 t at 40 m radius |
| --- | --- |
| 5 t / 16.16 m row | 2.20 t at 40 m radius |
| Difference | 2.20 - 1.73 = 0.47 t |
| Answer | NOTE: the supplied written-test choices do not include 0.47 t. |

7. Shorter Jib Comparison

| 50 m jib • 5 t row • 35 m | 2.04 t |
| --- | --- |
| 40 m jib • 5 t row • 35 m | 2.58 t |
| Difference | 2.58 - 2.04 = 0.54 t |
| Answer | B - 0.54 t |

8. Maximum Allowable Load

| Chart capacity | 45 m jib • 35 m radius • 5 t row = 2.52 t |
| --- | --- |
| Rigging | 0.21 t |
| Maximum load | 2.52 - 0.21 = 2.31 t |
| Answer | B - 2.31 t |

9. Between Listed Radii

| Actual radius | 32 m |
| --- | --- |
| Chart rule | Use next larger listed radius: 35 m |
| Gross suspended load | 2.10 + 0.10 = 2.20 t |
| 35 m capacity | 2.04 t |
| Result | No - overloaded by 0.16 t |
| Answer | C - No; use the 35 m capacity |

10. Load Chart + Hoist Mechanism

| Gross suspended load | 2.75 + 0.20 = 2.95 t |
| --- | --- |
| Structural capacity | 40 m jib • 5 t row • 30 m radius = 3.08 t |
| Structural result | Permitted; 3.08 - 2.95 = 0.13 t = 130 kg remaining |
| Hoist speed | Using the 5 t hoist configuration: 3 t rating = 26 m/min |
| Answer | YES - 130 kg remaining; highest hoisting speed = 26 m/min |

11. Relevant vs Irrelevant Information

| Relevant | 35 m jib • 5 t row • 30 m radius • 0.18 t rigging |
| --- | --- |
| Irrelevant to capacity lookup | 400 V / 50 Hz • 28 kVA • DVF 3 4 D6 |
| Chart capacity | 3.29 t |
| Maximum load | 3.29 - 0.18 = 3.11 t |
| Answer | 3.11 t maximum load weight, excluding rigging |

12. Final Multi-Step Problem

| Gross suspended load | 2.25 + 0.20 = 2.45 t |
| --- | --- |
| Maximum listed radius | 30 m |
| Capacity at 30 m | 2.45 t |
| Remaining capacity | 2.45 - 2.45 = 0 t |
| Hoist speed | Using the 5 t hoist configuration: 2.5 t rating = 30 m/min |
| Answer | 30 m maximum radius • 0 kg remaining • 30 m/min |

Instructor notes

• Question 6: direct chart values give 1.73 t versus 2.20 t, a 0.47 t difference. The current multiple-choice options do not contain 0.47 t, so this question should be corrected before use.

• Questions 10 and 12: the hoist-speed answers above use the 5 t hoist configuration. The written questions should explicitly state that hoist configuration so the mechanism lookup is unambiguous.