TEREX CTT 91-5
LOAD CHART & MECHANISMS TEST

Use only the supplied Terex CTT 91-5 manufacturer charts. Show calculations where required.

| Name: __________________________ | Date: __________________ | Score: ______ / 12 |
| --- | --- | --- |

1. Highest Permitted Hoisting Speed

The crane is equipped with the 18 AFC 25 hoist. Gross suspended load: 2.35 t. Hoist configuration: 2.5 t. What is the maximum permitted hoisting speed?

A. 34 m/min

B. 40 m/min

C. 51 m/min

D. 70 m/min

________________________________________________________________________________

2. Power Supply / Mechanism Identification

The crane is equipped with the 18 AFC 25 mechanism. According to the supplied mechanism chart, which power-supply specification is shown for this crane?

A. 208 V / 60 Hz

B. 230 V / 50 Hz

C. 400 V / 50 Hz or 460 V / 60 Hz

D. 600 V / 60 Hz

________________________________________________________________________________

3. Structural Capacity

Configuration: 50 m jib; 35 m working radius; 5 t load-chart row. Load: 1.85 t. Rigging: 0.15 t. Is the lift permitted?

A. Yes, with 40 kg remaining

B. Yes, with 140 kg remaining

C. No, overloaded by 40 kg

D. No, overloaded by 150 kg

________________________________________________________________________________

________________________________________________________________________________

4. Maximum Working Radius

Configuration: 50 m jib; 5 t load-chart row; gross suspended load: 2.40 t. What is the maximum LISTED working radius at which the load may be handled?

A. 16.16 m

B. 25 m

C. 30 m

D. 35 m

________________________________________________________________________________

________________________________________________________________________________

5. Correct Capacity Row

Configuration: 50 m jib; 40 m working radius. Use the 2.5 t row with the 30.63 m crossover value. What is the rated capacity?

A. 1.30 t

B. 1.73 t

C. 1.83 t

D. 2.50 t

________________________________________________________________________________

6. Same Crane — Different Configuration

At a 40 m working radius with the 50 m jib, compare the 2.5 t / 30.63 m row with the 5 t / 16.16 m row. What is the difference in rated capacity?

A. 0 t

B. 0.10 t

C. 0.33 t

D. 0.77 t

________________________________________________________________________________

________________________________________________________________________________

7. Shorter Jib Comparison

Using the 5 t rows, determine the difference in rated capacity at a 35 m working radius between the 50 m jib and the 40 m jib.

A. 0.44 t

B. 0.54 t

C. 0.64 t

D. 1.08 t

________________________________________________________________________________

________________________________________________________________________________

8. Maximum Allowable Load

Configuration: 45 m jib; 35 m working radius; 5 t row; rigging weight: 0.21 t. What is the maximum allowable LOAD weight, excluding rigging?

A. 2.10 t

B. 2.31 t

C. 2.52 t

D. 3.21 t

________________________________________________________________________________

________________________________________________________________________________

9. Between Listed Radii

Configuration: 50 m jib; 5 t row; actual working radius: 32 m; load: 2.10 t; rigging: 0.10 t. Is the lift permitted?

A. Yes — use the 30 m capacity

B. Yes — interpolate between 30 m and 35 m

C. No — use the 35 m capacity

D. No — use the 16.16 m crossover value

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

10. Load Chart + Hoist Mechanism

Configuration: 40 m jib; 5 t row; 30 m working radius; load: 2.75 t; rigging: 0.20 t. Determine (a) whether the lift is structurally permitted and (b) the highest permitted hoisting speed.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

11. Relevant vs Irrelevant Information

Configuration: 35 m jib; 5 t row; 30 m working radius; rigging: 0.18 t. Additional information: 400 V / 50 Hz power supply; 28 kVA average consumption; DVF 3 4 D6 trolley mechanism. What is the maximum allowable LOAD weight, excluding rigging?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

12. Final Multi-Step Problem

The operator must lift a 2.25 t load with 0.20 t of rigging using the 50 m jib and the 5 t row. Determine: (a) maximum listed working radius; (b) remaining capacity at that radius; and (c) maximum permitted hoisting speed for the gross suspended load.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Chart-reading rules: Include rigging in gross suspended load. Use the stated configuration. When an actual radius falls between listed radii, use the next larger listed radius; do not interpolate.